AWS3274: 3274 Control Unit Emulator

===================================

AWS3274 allows the P/390 to emulate a 3274 control unit and allows the P/390 to

support 3270 devices such as 3277, 3278, 3279, and 3287.

To use P/390 3270 support, you must have installed one of two 3270

emulator programs:

- Communications Manager/2 Version 1.11 for OS/2 WARP

- eNetwork Personal Communications Version 4.1 or higher

DEVICE TYPE: 3277, 3278, 3279, 3287, etc.

============

3274 Emulation using Communication Manager/2

============================================

AWS3274 can use the OS/2 Communications Manager/2 (CM/2) to emulate devices.

AWS3274 supports non-SNA DFT protocol and uses the CM non-SNA DFT 3270

Emulator.  CM supports a maximum of 5 DFT logical terminals.  Any one of

these can be configured as either a local session or a host session.  A local

session is a logical terminal active on the P/390.  A host session is a

logical terminal active on a host system connected via coax to the 3270

Connection Adapter in your PS/2.

The CM 3270 Emulator does not distinguish between a local and host LT

session.  Both types of LTs can be configured using CM's Advanced

Configuration.  Also, all functions available to a host LT session are also

available to a local LT session, for example, GDDM graphics, font selection,

cut and paste, etc.

The only difference between a local and host session is the way 3270 data

streams are processed.  For a host LT session, the data stream is transmitted

between the PS/2 and control unit over coax.  The coax is attached to a 3270

Connection card which uses a device driver (DFTDD.SYS) to interpret the data

streams and perform the appropriate inbound or outbound function.  For a

Local LT session, the P/390 session is physically in the same PS/2 as CM, so

no coax or 3270 Connection card is needed.  Instead, there are "hooks" in CM

to route 3270 data streams to and from the AWS3274 device manager, using CM's

outbound and inbound router function.

3274 Emulation using eNetwork Personal Communications (PC/3270)

===============================================================

(TCP/IP TN3270 protocol)

If CM/2 is not installed, all devices defined to the AWS3274 manager will

be changed to LAN3274 devices and handled by the LAN3274 manager via its

TCP/IP interface.  To determine if CM/2 is installed, a check will be made

for the DFT COAX Device Driver (DFTDD.SYS); if it is not found, then CM/2,

or at least the non-SNA DFT part that is required for Local LT sessions, is

assumed not to be installed.

Note: If CM/2 is installed, but you still wish to use the LAN3274 TCP/IP

support for local console support, you can do the following:

- Start the P/390 Configurator

- From the F2 "Update System Devices" menu, press F12 to go to the

"Device Manager Parm Edit/Display" submenu.

- Scroll down to the AWS3274 manager line.

- Add the option  /NOCM

Configuring a AWS3274 Device for CM/2

=====================================

To configure a AWS3274 Device, go to the Configurator F2 "UPDATE SYSTEM

DEVICES" menu and enter the real address of a 3278 or equivalent device in

your system I/O configuration.  For VM/ESA (370 feature) or VM/SP this would

include those entries of DMKRIO.ASM that have a DEVTYPE of 3278 (for example)

and a CUTYPE of 3274, or 3174 in DMKRIO.ASM.

In the Mgr field, enter the Manager code for AWS3274 (usually 3).

In the FN/P field at the right, you may specify the name of the window.  By

default, CM titles its LT windows "XXXX - A - 3270 Emulator", where "XXXX" are

the first 4 characters of the LTs session ID.  For Local LTs the default LT

window title is changed to "P/390 Session".  You can override this default by

specifying a new window title in the FN/P field for each 3270 device address.

Use the Configurator "Update 3270 LT Sessions (F7) screen to specify which of

the 5 CM LTs will be Local and which will be Host LT sessions.

Note: You can only assign as "host" those LTs that would normally be available

as sessions on your coax.  Thus, if your coax is configured for one host

session, only the first LT can be specified as "host".  If your coax is

configured for two host sessions then either LT 1 or LT 2 may be

specified as "host", but NOT LT 3, 4 or 5.

Miscellaneous Information for CM/2

==================================

Device Addresses managed by AWS3274 are assigned to local LT sessions

sequentially, starting with the lowest address first.  Thus if you have AWS3274

device address 200 and 201 defined, and CM LTs B and D are local LTs (thus A, C

and E are Host LTs) the device address are assigned as follows:

A:  Host  LT 1

B:  Local LT 2 = Device 200

C:  Host  LT 3

D:  Local LT 4 = Device 201

E:  Host  LT 5

As mentioned earlier, your host coax MUST be configured for FIVE sessions

in order for LT 1, LT 3, and LT 5 to be active.

When IPLing the VM operating system, the system console can be brought

up on a 3270 device if defined via the appropriate RIOGEN statement in

DMKRIO (370 feature) or SYSTEM CONFIG (ESA feature).

Configuring a AWS3274 Device for TCP/IP

=======================================

>>> NOTE: The only supported TN3270 emulator for local P/390 consoles is

>>>       "eNetwork Personal Communications" version 4.1 or higher.

If CM/2 is not installed, or if /NOCM is specified, all AWS3274 defined

devices will be handled as if they were defined as LAN3274 devices (i.e. they

will be managed by the LAN3274 device manager.  See LAN3274.DOC for details).

Each converted AWS3274 device will be assigned a security IP address of

127.0.0.1 which is the loopback address.  This will allow allow these

devices to act as local consoles even if no LAN adapter is present.

--> You MUST enable the loopback interface in TCP/IP Configuration:

a. Type TCPCFG to open the TCP/IP Configuration notebook.

b. Click on "loopback interface" on the "Configure Network Interface" page

c. Make sure the "Enable Interface" box is checked.

d. Make sure the IP address is 127.0.0.1

e. Double click on the upper left and Save Changes

f. Shutdown and reboot OS/2 to make changes effective

Note: Unlike CM/2 sessions, there is no fixed association between the PC/3270

"Short Session ID" (e.g. A) and the S/390 device address (e.g. 200).

Thus, if you start more than one local PC/3270 session and then IPL,

it is impossible to predict which session the System Operator will

appear on. Also note that window titles via the DEVMAP FN/P field or

the CMS LTRENAME command are not supported with PC/3270. The OS/2

LTRENAME command is supported, as it takes short session IDs.

Miscellaneous Information

=========================

In general, Local 3270 sessions must be started *BEFORE* IPLing any

operating system on the P/390.  The one exception is the 3215 teletype

console, which is only supported by VM/370 and standalone programs such as

DDR and ICKDSF.  Most other host systems, OS/390, MVS/ESA, VM/ESA, and

VSE/ESA, require a 3270 console to be present when the process starts.